Patent 5: AVR

1 paper, 2 papers, Paper F (low power), Paper G (high power)

A Full State Feedback Controller for Dynamic Capacitive Wireless Power Transfer Systems

Ben Liao, Sophia Lin, Dheeraj Etta, Khurram K. Afridi

Department of Electrical and Computer Engineering Cornell University

Email: {bhl47, sl2383, dke27, kka34}@cornell.edu

Abstract—This paper presents a novel full-state feedback (FSF) controller for compensating changing coupling variations in dynamic capacitive wireless power transfer (WPT) systems utilizing an active variable reactance (AVR) rectifier. The AVR rectifier enables continuous compensation, allowing the WPT system to maintain high-efficiency power transfer at a fixed frequency by appropriately distributing power between its two branches. Conventional PID control is suboptimal for the AVR rectifier due to the higher-order system dynamics, nonlinearities, and cross-coupling between control loops. In contrast, the FSF control strategy presents an optimal and robust way to control higher order multi-input multi-output systems like the AVR rectifier. A state-space model is developed for the AVR rectifier, and the FSF controller is designed based on this model. To validate the proposed approach, a 13.56 MHz 25-W capacitive WPT system with an AVR rectifier is built and tested. The controller’s effectiveness is demonstrated through both simulations and experiments on the hardware prototype.

# I. INTRODUCTION

Wireless power transfer (WPT) is becoming increasingly important and has the potential to accelerate the widespread adoption of electric vehicles (EVs), especially amongst vehicles that routinely travel along fixed routes such as buses and warehouse forklifts [1], [2]. Capacitive WPT systems can be lighter, cheaper, and smaller than comparable inductive WPT systems due to the absence of ferrite cores [3]. The ability of capacitive WPT systems to transfer high power at high efficiency has been demonstrated, thus proving the viability of this technology [4].

One main challenge of WPT systems is the extreme sensitivity to the value of the coupling capacitance or mutual inductance between the receiver and the transmitter [5]. To increase the practicality of WPT systems in dynamic applications, it is desirable to design and control them to tolerate coupling variations while maintaining full power transfer at high efficiency [6]–[9]. The recently proposed active variable reactance (AVR) rectifier addresses these challenges and efficiently provides variable compensation in high-frequency WPT systems [10]. However, a robust controller is needed in order to perform this compensation automatically and with a sufficiently high bandwidth to allow for in-motion dynamic EV charging. A conventional PID control-based decoupled dual-loop strategy has been proposed to compensate for changing coupling capacitance while maintaining the desired power transfer [11], [12]. However, such approaches are not well-suited to optimally control the AVR rectifier due to the higher-order system dynamics, nonlinearities, and crosscoupling between the two control loops.

This work presents a novel time-domain solution for the control of the AVR rectifier with sufficiently high bandwidth and robustness to operate the system dynamically. First, a model of the AVR rectifier is derived in state space. Then, a state-space control strategy is proposed, and an optimal full state feedback (FSF) linear quadratic regulator (LQR) controller is designed and validated in simulation. Finally, a prototype 13.56-MHz, 25-W capacitive WPT system with an AVR rectifier and a dynamically variable coupling capacitance is built and its operation demonstrated with the proposed control strategy.

# II. DYNAMIC CAPACITIVE WIRELESS POWER TRANSFER SYSTEM WITH AN AVR RECTIFIER

Figure F.1 presents the circuit schematic of the dynamic capacitive WPT system with the AVR rectifier. A DC input bus is connected to a full-bridge inverter. Power then flows through the tank formed by Lcomp and Cs. Cs is the coupling capacitance between the roadway-side and vehicle-side coupling plates; Lcomp is designed such that the impedance of the series-resonant LC tank is zero when Cs assumes its nominal value of Cs,nom.

The AVR rectifier has two branches: a top branch containing a boost converter attached to an inductive reactance +jX, and a bottom branch containing a buck converter attached to a capacitive reactance -jX. By independently varying the amount of power processed in each branch, the AVR rectifier can maintain the desired constant output power (Pout) while compensating for coupling variations by modulating the impedance seen at the input of the AVR rectifier (zr). Throughout this paper, the following notation is used: pout is the time-varying measured variable, Pout is the constant nominal DC value, and pout is the time-varying small-signal variable. They are related as poutt≡Pout+poutt. Equations 1a-1b give pout and zr as a function of the large-signal circuit states as labeled in Fig. F.1 [10]: krec is a gain associated with the half-bridge rectifiers under sinusoidal approximation, and X is the differential reactance ±jX attached to the two branches. When the coupling capacitance Cs changes by an amount ΔCs, Eqn. 1b is utilized to adjust the operation of the AVR rectifier such that xr perfectly compensates ΔCs while maintaining rr=Rr.

(1b)

(1a)

This also guarantees pout=Pout, as well as resistive loading on the inverter.

pout=p1+p2=v1iL1+voutiL2

zr=Rezr+j⋅Imzr=rr+jxr=krec2v12v22+v1voutiL1iL2X2krecv1v22iL1+voutv12iL2+jXv1iL1v22-voutiL2v12v1iL1v22+voutiL2v12

A frequency-based controller has been proposed in [12] which utilizes two control loops: one maintains pout=Pout, and the other maintains rr=Rr. The two loops run at the same frequency, but use constant decoupling gains to minimize cross-coupling between the loops. However, the controller does not have sufficient performance to operate the system in dynamic charging scenarios. The AVR rectifier is a highly nonlinear, heavily-coupled multi-input multi-output (MIMO) system; FSF controllers are typically able to handle such plants more robustly than conventional frequency-based controllers. The FSF controller designed in this work follows the same control strategy of attempting to hold pout and rr constant at their nominal values Pout and Rr.

III. STATE SPACE MODELING OF THE AVR RECTIFIER

In this section, a model for the AVR rectifier is developed in state space using a method similar to that employed in [13], [14]. Sec. IV uses this model for the design of a robust FSF controller. Figure F.2 contains the small-signal linearized model of the AVR rectifier. Of note is the inclusion of the input capacitance on both of the converters; these capacitors are necessary in modeling the dynamics of the converter as the input voltages v1 and v2 vary.

In general, a continuous-time state-space model can be written as ddtx=Ax+Bu, where x is the state vector, u is the input vector, A is the system dynamics matrix, and B is the input dynamics matrix. The AVR rectifier state-space model is derived from its small-signal model, so the state vector x contains the small-signal voltages and currents v1, v2, iL1, iL2, vout, as labeled in Fig. F.2. The input vector  u contains the small-signal control handles d1 and d2, which correspond to the small-signal duty cycles of the boost and the buck converters, respectively.

First-order differential equations for ddtx are written using time-domain circuit analysis techniques. These are arranged into a matrix, resulting in a valid five-state un-augmented state space model of the AVR rectifier. However, the controller design requires additional augmented integral states z1≡0tpoutτdτ and z2≡0trrτdτ and  which track the integral of the error in output power poutt and error in the real part of the input impedance rrt, respectively.

To incorporate z1 and z2 into the state-space model, the quantities ddtz1=pout and ddtz2=rr must be written as linear combinations of the elements of x. This can be done by taking partial derivatives of Eqns. 1a-1b, with results shown in Table I. Thus, an augmented state vector xaug=xz1z2T can be constructed, and the complete state-space model is presented in Eqn. 2. This model is the first significant contribution of this paper.

ddtvoutv1v2iL2iL2z1z2=-1RCout00D1'Cout1Cout00000-1C10000000-D2C200-D1'L11L100000-1L20D2L20000-∂pout∂vout-∂pout∂v1-∂pout∂v2-∂pout∂iL1-∂pout∂iL200-∂rr∂vout-∂rr∂v1-∂rr∂v2-∂rr∂iL1-∂rr∂iL200voutv1v2iL2iL2z1z2+-IL1Cout0000-IL2C2VoutL100V2L20000d1d2

(2)

IV. FULL STATE FEEDBACK CONTROLLER DESIGN AND SIMULATION VALIDATION

Figure F.3 shows the complete block diagram of the system under closed-loop control. The controller design consists of finding a gain matrix K such that setting the input as u=D-Kxaug results in the closed-loop system driving pout and rr to their nominal values Pout and Rr regardless of the value of Cs. To find an optimal K, heuristic techniques presented in [15] are used to design a linear quadratic regulator (LQR) with reasonable values for the diagonal elements of the Q and R cost-defining matrices. Then, the lqr() function in MATLAB is utilized to solve the corresponding Algebraic Riccati Equation for the optimal gain matrix K.

Q is diagonal, with the elements along the diagonal quantifying the cost assigned to each element of the augmented state vector  xaug. R is also diagonal, with the elements along the diagonal quantifying the cost assigned to each element of the input vector u. Since the controller is only required to drive the two augmented states z1 and  z2 to zero and should not respond to DC deviations in any of the elements of x, only the weights corresponding to z1 and z2 should be nonzero in Q. Both diagonal elements of R should be nonzero. The structure of these matrices is shown below:

Q=05x505x202x5qz100qz2 		 R=rd100rd2

Tuning the controller consists of finding a set of strictly positive values for qz1, qz2, rd1, and rd2 which results in the desired performance. Bryson’s Rule for tuning Q and R is used as a starting point for the tuning process.

A model is built in Simulink and PLECS to validate the proposed controller. Figure F.4 shows the control objectives pout and rr being controlled to their target values of Pout=200 W and Rr=50 Ω respectively, while the coupling reactance is disturbed by an amount ΔXs≡12πfs1Cs,nom-1Cs , where fs is the switching frequency, and Cs,nom is the nominal coupling capacitance. The impedance seen by the inverter must be near-resistive for the converter to operate at peak efficiency. Figure F.5 shows the inverter output voltage (vinv) and inverter output current (iinv) waveforms under a misalignment condition with and without the controller. The proposed controller is enabling the AVR rectifier to compensate for a change in coupling reactance, as shown by the inverter output voltage and current waveforms being nearly in-phase. The design of the FSF controller for the AVR rectifier and simulation results showing its effectiveness are the second significant contribution of this work.

# V. HARDWARE AND EXPERIMENTAL RESULTS

The proposed controller is implemented on a prototype 13.56 MHz, 25-W capacitive WPT system equipped with an AVR rectifier and an adjustable coupling capacitance. Figure F.6 shows the hardware prototype and experimental setup with annotations labeling important components, whose values are listed in Table II. The system is tuned to operate in the nominal condition, with the inductor Lcomp compensating for the coupling capacitance Cs at the desired operating frequency. Then, the system is put in a misaligned condition, where degraded performance is observed. Finally, the controller is activated, which brings the system back to full operating power with resistive loading on the inverter.

Figure F.9 shows the duty cycles to the two converters changing as a result of the control action.

Figure F.8 shows the effect of the control action on the AVR branch currents. Note that the sum of the currents through the two branches has increased as a result of the control action, showing that the full nominal output power has been restored. The current ripple changes because the duty cycles of the two converters change with control action.

Figure F.10 shows the effect of the control action on the inverter output voltage and current. Note that there is a small but noticeable phase difference between the voltage and current before the controller is activated, which may cause the inverter to lose soft switching at higher power. After the control action is taken, the voltage and current are perfectly in phase. Also note that the magnitude of the inverter output current decreases; this shows that the control action is reducing the circulating currents in the resonant tank by ensuring the AVR rectifier and tank present a resistive load to the inverter.

Figure F.7 shows the transient behavior of the converter when the controller is activated. Note that the three voltages settle to new values after the control action is taken, and that the settling time is around 1 millisecond — more than enough bandwidth to track a vehicle moving over a capacitive WPT charging pad. The controller implementation and experimental results showing controller effectiveness comprise the third significant contribution of this work.

# VI. CONCLUSION AND FUTURE WORK

This paper presents a novel state-space model and full state feedback (FSF) controller for capacitive WPT systems equipped with AVR rectifiers. The proposed controller allows the system to transfer power efficiently as the coupling capacitance between the transmitter and receiver is varied. A 13.56-MHz, 25-W capacitive WPT system with an AVR rectifier and a dynamically adjustable coupling capacitance was assembled. The proposed controller was validated in simulation and on the hardware prototype.

# VII. ACKNOWLEDGEMENTS

The author would like to acknowledge fellow High Frequency Power Electronics Group members Xuancen Wu and Abdullah Saboor for their ideas and assistance, as well as Prof. Douglas MacMartin of the Department of Mechanical and Aerospace Engineering at Cornell University for his insights and suggestions.

# REFERENCES

- J. M. Miller, P. Jones, J.-M. Li, and O. C. Onar, “Ornl experience and challenges facing dynamic wireless power charging of ev’s,” IEEE Circuits and Systems Magazine, vol. 15, no. 2, pp. 40–53, 2015.

- B. Regensburger, A. Kumar, S. Sinha, and K. Afridi, “Highperformance 13.56-mhz large air-gap capacitive wireless power transfer system for electric vehicle charging,” in 2018 IEEE 19th Workshop on Control and Modeling for Power Electronics (COMPEL), pp. 1–4, 2018.

- G. A. Covic and J. T. Boys, “Modern trends in inductive power transfer for transportation applications,” IEEE Journal of Emerging and Selected Topics in Power Electronics, vol. 1, no. 1, pp. 28–41, 2013.

- S. Maji, D. Etta, and K. K. Afridi, “A high-power large air-gap multimhz dc-dc capacitive wireless power transfer system for electric vehicle charging,” in 2023 IEEE Wireless Power Technology Conference and Expo (WPTCE), pp. 1–6, 2023.

- M. Budhia, G. A. Covic, and J. T. Boys, “Design and optimization of circular magnetic structures for lumped inductive power transfer systems,” IEEE Transactions on Power Electronics, vol. 26, no. 11, pp. 3096–3108, 2011.

- Y. Lim, H. Tang, S. Lim, and J. Park, “An adaptive impedancematching network based on a novel capacitor matrix for wireless power transfer,” IEEE Transactions on Power Electronics, vol. 29, no. 8, pp. 4403–4413, 2014.

- T. C. Beh, M. Kato, T. Imura, S. Oh, and Y. Hori, “Automated impedance matching system for robust wireless power transfer via magnetic resonance coupling,” IEEE Transactions on Industrial Electronics, vol. 60, no. 9, pp. 3689–3698, 2013.

- J. Choi, J. Xu, R. Makhoul, and J. M. R. Davila, “Implementing an impedance compression network to compensate for misalignments in a wireless power transfer system,” IEEE Transactions on Power Electronics, vol. 34, no. 5, pp. 4173–4184, 2019.

  - Kamineni, M. J. Neath, G. A. Covic, and J. T. Boys, “A mistuningtolerant and controllable power supply for roadway wireless power systems,” IEEE Transactions on Power Electronics, vol. 32, no. 9, pp. 6689–6699, 2017.

- S. Sinha, A. Kumar, B. Regensburger, and K. K. Afridi, “Active variable reactance rectifier—a new approach to compensating for coupling variations in wireless power transfer systems,” IEEE Journal of Emerging and Selected Topics in Power Electronics, vol. 8, no. 3, pp. 2022–2040, 2020.

- S. Sinha and K. K. Afridi, “Closed-loop control of a dynamic capacitive wireless power transfer system,” in 2019 20th Workshop on Control and Modeling for Power Electronics (COMPEL), pp. 1–6, 2019.

- S. Maji, Y. Cao, D. Etta, and K. K. Afridi, “An intelligent control methodology for in-motion dynamic capacitive wireless power transfer systems,” in 2024 IEEE Workshop on Control and Modeling for Power Electronics (COMPEL), pp. 1–7, 2024.

- J.C. Mayo-Maldonado, J.C. Rosas-Caro, R.Salas-Cabrera, A.Gonzalez-Rodriguez, O. F. Ru´ ´ız-Mart´ınez, R. Castillo-Gutierrez,´ J. R. Castillo-Ibarra, and H. Cisneros-Villegas, “State space modeling and control of the dc-dc multilevel boost converter,” in 2010 20th International Conference on Electronics Communications and Computers (CONIELECOMP), pp. 232–236, 2010.

- R. W. Erickson and D. Maksimovic, Fundamentals of Power Electronics. Springer, 2004.

- R. Williams and D. Lawrence, Linear State-Space Control Systems. Wiley, 2007.